Iowa State University

# Emergency Tractor Beacon

A co-operative project between the Electrical and Computer Engineering Department and the Department of Industrial Design

John Pritchard, David Ringholz, Mani Mina, Jamie Horwitz, Michael Tschampl, Mitchell Hinrichsen

7/9/2012

The purpose of this document is to keep a working record of the motivation, design, specification, and changes of the Emergency Tractor Beacon project.

Contents

Introduction	2

Solution Outlook	2

Detailed Solutions	2

Prototype	3

Systems Level Design	3

Electronics Design	4

Software Design	4

Mechanical Description	7

Specifications	7

Level of Effort	7

Cost Analysis	7

Appendix	8

Prototype Software	8

Arduino Code	8

MATLAB Code	9

Index	Error! Bookmark not defined.

Figure 1: Prototyped wireless transceivers	3

Figure 2: Prototype systems-level diagram	4

Figure 3: Prototype electronics diagram	4

Figure 4: Prototype Arduino software flow chart	5

Figure 5: Prototype MATLAB software flow chart	6

Table 1: Brief outlook of proposed solutions	2

Table 2: Prototype cost analysis	7

# Introduction

It is a continued problem that people are fatally injured due to tipping or even rolling tractors built after the early 1970’s. No general solution has been proposed to help aid in the event of such an occurrence, until now. This document proposes three solutions, each with their respective advantages and disadvantages, which signal local authorities in the event of a tip or roll to quickly respond. In addition, a working prototype is described and presented, showing the proof of concept.

The structure of this document is designed to provide a quick overview of the proposed solutions, so the reader can choose which is appropriate for their application. Each solution is then discussed in detail, presenting system descriptions, components, requirements, specifications, level-of-effort, and cost analysis.

# Solution Outlook

This section briefly discusses and compares the proposed solutions, highlighting the most important design tradeoffs.

| Specification | Prototype | Basic | Intermediate | Advanced |
| --- | --- | --- | --- | --- |
| Power Requirement | Low | Low | Medium | High |
| Com Distance* | 200m | >3mi. | Global | Global |
| Usability | Easy | Easy | Moderate | Cumbersome |
| Sys Complexity | Simple | Simple | Moderate | Complicated |
| Maintenance | N/A | Low | Moderate | Very High |
| Level-of-Effort | ~2 wks. | ~1-2 mo. | ~ 3-4 mo. | 8-12 mo. minimum |
| Reliability | N/A | Moderate | High | High |
| Multidisciplinary Impact | N/A | Low | Moderate | Very High |
| Cost | ~$250 | ~$400 | ~$800 | >$4000 |

*Com Distance refers to how far the wireless transmission communicates. Upon successful wireless transmission, authorities will be notified over Ethernet (the internet).

Table 1: Brief outlook of proposed solutions

# Detailed Solutions

This section describes each proposed solution in detail. The following are provided for each solution:

- Systems-level design

- Electronics design

- Software design

- Mechanical descriptions

- Specifications

- Level-of-effort (LOE)

- Cost analysis

The systems-level design describes how all main components are connected, and how and when information is transferred. It is the description of how the system works at the highest level.

The electronics design describes the hardware involved, generally from a high level perspective. Low level descriptions will be provided when custom circuitry is designed.

The software design describes the software that will need to be written to accomplish tasks given by the specifications and requirements.

The mechanical descriptions provide all physical characteristics of components used including dimension, material, and durability.

The specification description provides a list of capabilities of the solution at hand.

The LOE descriptions provides how much time and resources are required to realize the solution

The cost analysis provides a list of all components, how to purchase them, how much they cost, and alternatives for cost reduction of applicable.

## Prototype

The prototype is a small-scale wireless system (Figure 1) in which the transmitter transmits a warning if the angle of the tractor (with respect to the horizon) reaches beyond a defined threshold.

Figure 1: Prototyped wireless transceivers

### Systems Level Design

In the prototype, one transceiver is connected to a PC and awaits the wireless transmission of the warning signal. The other transceiver is attached to the tractor and monitors the tractor’s orientation relative to the horizon. If the tractor angles beyond a defined threshold, the attached transceiver will immediately transmit a warning, which will be interpreted by a PC computer on the other end (see Figure 2)

Transciever

PC

Transciever

Tractor

Figure 2: Prototype systems-level diagram

### Electronics Design

In this section, each transceiver is defined.

The transceiver attached to the PC contains the following:

- Arduino (electronic controller)

- Xbee (transceiver module)

The transceiver attached to the tractor contains the following:

- Arduino (electronic controller)

- Xbee (transceiver module)

- Electronic Gyroscope (gyro)

The Arduino is a common electronics platform that allows for fast prototyping and interfacing of other electronics sensors, communication devices, etc. The Arduino can handle the information provided by the gyro and the Xbee.

The gyro measures the angular velocity of a given axis, and outputs a voltage that is representative of this rotation. The Arduino converts this voltage to a digital value, and manipulates the data to extract degree information.

The Xbee is an advanced wireless device that can easily interface with the Arduino controller. The range on the particular devices used is near 200 meters line-of-sight.

A connection diagram of this system is provided:

Arduino

Xbee

Arduino

Xbee

PC

End-User

Tractor

Gyro

Figure 3: Prototype electronics diagram

### Software Design

This section discusses the design of the software. It is split into two sections, since two programming environments were used: Arduino and MATLAB. The Arduino software design controls the sensor and wireless information handling. The MATLAB software design controls external notification (Google maps and cellular notification).

#### Arduino Software

The flow chart describing this software can be found below. The code is given in the appendix.

Figure 4: Prototype Arduino software flow chart

#### MATLAB Software

The flow chart describing this software can be found below. The code is given in the appendix.

Figure 5: Prototype MATLAB software flow chart

### Mechanical Description

Each device is 2.5in x 2.5in x 2in.

### Specifications

The device can communicate wirelessly about 200yds with a 6V power supply.

This specification is theoretical, has yet to be verified.

### Level of Effort

This project took about 4 weeks to define and create in full. $250 paid for all components and software (with the exception of the MATLAB license, which was a student license).

### Cost Analysis

This section describes the cost of the components and supplies.

| Tractor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Description | Cost | Qty | Tot | Link |
| Arduino | controller | $ 35.00 | 1 | $ 35.00 | http://www.sparkfun.com/products/11021 |
| Proto Shield | controller | $ 15.00 | 1 | $ 15.00 | http://www.sparkfun.com/products/7914 |
| Xbee | wireless | $ 26.00 | 1 | $ 26.00 | http://www.sparkfun.com/products/10415 |
| Xbee Explorer | dongle | $ 25.00 | 1 | $ 25.00 | http://www.sparkfun.com/products/8687 |
| Gyro | LPY503AL | $ 30.00 | 1 | $ 30.00 | http://www.sparkfun.com/products/9424 |
| Battery Holder | 4x AA Barrel connector | $ 2.50 | 1 | $ 2.50 | http://www.sparkfun.com/products/9835 |
| Total |  |  |  | $ 133.50 |  |

| End User |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Description | Cost | Qty | Tot | Link |
| Arduino | controller | $ 35.00 | 1 | $ 35.00 | http://www.sparkfun.com/products/11021 |
| Proto Shield | controller | $ 15.00 | 1 | $ 15.00 | http://www.sparkfun.com/products/7914 |
| Xbee | wireless | $ 26.00 | 1 | $ 26.00 | http://www.sparkfun.com/products/10415 |
| Xbee Explorer | dongle | $ 25.00 | 1 | $ 25.00 | http://www.sparkfun.com/products/8687 |
| Total |  |  |  | $ 101.00 |  |

| Grand Total | $ 234.50 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Table 2: Prototype cost analysis

# Appendix

## Prototype Software

### Arduino Code

float Aval1 = 0;  //define first analog read value

float Aval2 = 0;  //define second analog read value

float Vref = 0;

float maxV = 0;

float minV = 0;

float dVh = 0;

float dVl = 0;

float scaleFactor = 0; //scaling factor used in calibration

float zeroPoint = 233.55;  //define digital value that represents a stable device

float DtoA = 0.004883;  //define D to A conversion factor

float VtoDegPerSec = 0.033;  //define voltage to degree per second conversion factor

float DegPerSec = 0;  //define the angular velocity

float deg = 0;  //define initial degree (assume level initially)

float ddeg = 0;  //define incremental degree;

float dt = .004;              //define the sample period (0.0001s)

long ms1 = 0;

long ms2 = 0;

float numRead = 0;

float compensation = 34.56;

float AvalAvg;

float Aval_tmp;

float Aval;

void setup(){

pinMode(15,INPUT);  //set z-axis analog input to INPUT

pinMode(16,INPUT);  //3.3V reading, for calibration

pinMode(17,INPUT);  //GND reading, for calibration

pinMode(18,INPUT);  //VREF reading, for calibration

Serial.begin(9600);  //start up the serial at 9600baud

scaleFactor = 0.345;

}

void loop(){

ms1 = millis();

ms2 = ms1;

while((ms2-ms1)<dt*1000){

Aval_tmp = analogRead(1);

if(Aval_tmp >=468){

Aval_tmp = 468;

}

Aval = Aval + Aval_tmp;

numRead++;

ms2 = millis();

}

//calculate the deg per second

AvalAvg = Aval/numRead;

DegPerSec = ((((AvalAvg)-zeroPoint)*DtoA))/(VtoDegPerSec); //calculate angular velocity

//update the current degree

deg = deg + DegPerSec;

if(deg > 4000){

Serial.println("41.772112,-93.646774");

while(1){} //stop, infinite loop

}

if(deg < -4000){

Serial.println("41.772112,-93.646774");

while(1){} //stop, infinite loop

}

Aval = 0;

numRead = 0;

}

### MATLAB Code

#### Main Script

clear all

close all

comNum = input('Which COM port (e.g. 14)?: ','s');

comNum = ['COM',comNum];

s1 = serial(comNum,'BaudRate',9600);

fopen(s1);

disp('Proceeding to receive data...');

while(1)

if (s1.BytesAvailable > 0)

if (s1.TransferStatus == 'idle')

pause(0.01);

BA = s1.BytesAvailable;

q = fread(s1, s1.BytesAvailable, 'char');

q = q';

fprintf('%s\n', char(q));

break;

end

end

end

fclose(s1)

delete(s1)

clear s1

i = 1;

lat_val = [];

lon_val = [];

while(q(i) ~= 44 && i < 10)

lat_val(i) = char(q(i));

i = i + 1;

end

i = i+1;

while(i < 21)

lon_val(i-10) = char(q(i));

i = i + 1;

end

lat_val = char(lat_val);

lon_val = char(lon_val);

cell_notify(lat_val, lon_val)

#### Cellular Function

function cell_notify(lat, lon)

latitude = lat;

longitude = lon;

myaddress = 'etalert.notify@gmail.com';

mypassword = 'ETPhoneHome';

setpref('Internet','E_mail',myaddress);

setpref('Internet','SMTP_Server','smtp.gmail.com');

setpref('Internet','SMTP_Username',myaddress);

setpref('Internet','SMTP_Password',mypassword);

props = java.lang.System.getProperties;

props.setProperty('mail.smtp.auth','true');

props.setProperty('mail.smtp.socketFactory.class','javax.net.ssl.SSLSocketFactory');

props.setProperty('mail.smtp.socketFactory.port','465');

prefix = ' https://maps.google.com/maps?q=';

comma = ',';

sentence = 'Potential rollover detected. Immediately assist the location ';

web_location = [prefix,latitude,comma,longitude];

body = [sentence,latitude,comma,longitude,web_location];

sendmail('7123890381@vtext.com',body)

web(web_location, '-browser')

end